# Editable tables

Table 1. Evidence supporting the serviced-hardware threat model

| Evidence domain | Demonstrated capability | Transfer to serviced hardware | Boundary |
| --- | --- | --- | --- |
| Human mobility traces | Sparse spatiotemporal points can uniquely characterize individuals [4]. | Repeated vehicle events may be joined with schedules or access data. | Direct for mobility data; identity requires auxiliary information. |
| Shared micromobility | Longitudinal vehicle activity can be reconstructed despite identifier changes [5]. | Fleet telemetry can preserve continuity around a facility. | Direct for studied services; architecture differs by operator. |
| Battery side channels | Electric-vehicle battery consumption revealed driver, style, occupancy, and habitual endpoints [11]. | Battery or controller data may expose recurring operational patterns. | Direct for electric-vehicle datasets; conditional on resolution and access. |
| Electrical load inference | Power signatures can reveal appliance use and behavior [12,13]. | Supports inference from non-positional telemetry. | Mechanism analogy; not proof for every device. |
| BMS history | Health and forensic analysis use cycle, voltage, current, and temperature history [14-16]. | Retained records may remain informative during service or return. | Data depth and storage location are implementation-specific. |
| Lifecycle custody | Supply-chain and IoT guidance treats access, logging, support, and sanitization as lifecycle controls [19-23]. | Maintenance, recall, and disposal require explicit assurance. | Normative guidance; effectiveness requires verification. |

Table 2. Lifecycle controls and auditable evidence

| Stage | Control objective | Minimum requirement | Evidence of compliance |
| --- | --- | --- | --- |
| Procurement | Establish visibility before deployment | Data inventory; component and software provenance; sub-processor and jurisdiction list | Architecture diagram; software bill of materials; supplier attestations; contract schedules |
| Deployment | Bind device to approved context | Hardened configuration; disabled unnecessary interfaces; approved identifiers and zones | Configuration record; acceptance test; asset registration |
| Operation | Minimize and contain telemetry | Purpose limitation; local processing where feasible; bounded retention; least privilege | Field-level schema; retention test; access logs; deletion evidence |
| Maintenance | Control diagnostic and physical access | Vetted staff; authorized tools; logged sessions; chain of custody; tamper evidence | Work order; access log; tool inventory; custody receipt |
| Recall/return | Prevent emergency return from bypassing security | Data-bearing component check; approved recipient; sanitize, escrow, or controlled retention | Recall playbook; transfer authorization; sanitization or legal-hold record |
| Second life/disposal | Remove residual data and credentials | Verified sanitization of all components; key revocation; controlled recycler | Sanitization certificate; revocation log; recycler audit |
| Continuous assurance | Detect changes in supplier or architecture | Incident notice; vulnerability support; material-change approval; periodic reassessment | Update records; support dates; audit report; change notifications |